Pharmeuropa SOW 2 / 36.02 - BAC - 2025-06-04 08:48:18

2.9.17. Test for extractable volume of parenteral preparations

PA/PH/Exp. 12/T (20) 47 ANP R1

12

BRIEFING NOTE

This draft corresponds to Revision 2 Stage 2 ver.1 (based on the Pharmacopoeial Discussion Group (PDG) working procedure) within the pharmacopoeial harmonisation process. The co-ordinating pharmacopoeia is the European Pharmacopoeia. The original text submitted to the PDG is published in the Pharmacopoeial harmonisation section.

Compared to the monograph published in the 11th Edition of the Ph. Eur., the following changes are proposed:

– The testing procedure has been updated, eliminating the requirement for a specific number of test samples to conduct the test. Due to different tolerances regarding the fill volume during production and the performance of the filling equipment, the number of containers to be tested is to be determined using a suitable statistical approach.

– The requirement for the use of a specific syringe and needle has been removed. The size of syringe and needle to use is to be read from the product label to better mimic clinical practice.

– pooling of volumes is no longer allowed when measuring the extracted volume from small containers, since it is important that each container fulfils this requirement;

– direct emptying is no longer described for small-volume parenterals in single-dose containers with a capacity of 10 mL or more to better mimic clinical practice.

XXXX:20917

2.9.17. Test for extractable volume of parenteral preparations(1)

Extractable volume of parenteral preparations, test for (2.9.17.)

Parenteral preparations, test for extractable volume of (2.9.17.)

Test for extractable volume of parenteral preparations (2.9.17.)

Each container of an injection or infusion contains sufficient overfill to allow withdrawal of the labelled quantity of preparation. Such withdrawal must be performed according to the instructions to the patient. The overfill must be justified and agreed by the competent authority considering the product characteristics.

The test applies to preparations supplied in single-dose containers as well as in multidose containers, independently of the volume. For the preparations supplied in multidose containers labelled to yield a specific number of doses of a stated volume the entire volume of the container is to be assessed.

Suspensions and emulsions are shaken before withdrawal of the contents and before the determination of the density. Oily and viscous preparations may be warmed according to the instructions on the label, if necessary, and thoroughly shaken immediately before removing the contents. The contents are then allowed to cool to 20-25 °C as required for the volumetric measurement. before measuring the volume.

Sterile solid formulations must be reconstituted according to the instructions on the label before removing the contents. Contents are then to be measured following one of the procedures described below.

PROCEDURE

Determine an appropriate number of containers to be tested, based on the number of units available and a suitable statistical approach

Following the instructions on the product label, withdraw the contents of each container selected into a suitable, dry container, such as a syringe or a dry tared beaker. If required in the instruction to the patient, expel any air bubbles from the syringe and needle, taking care not to spill out any product. The size of the syringe and the needle used is as indicated in the instructions to the patient to better mimic clinical practice. For example, when withdrawing product from a vial, use a syringe which is fitted with a 21-gauge needle not less than 2.5 cm in length.

In case of large volume preparations the content of the container could be emptied.

Proceed with the evaluation of the extractable volume as follows. Method A is the preferred method, whenever feasible.

Method A: Discharge the withdrawn (?) contents of the container into a tared dry container and exactly weigh the extracted volume. The volume of the contents may subsequently be calculated as the mass in grams divided by the density.

Method B: Discharge the withdrawn (?) contents of the container into a calibrated dry cylinder of such a capacity that the volume to be measured occupies at least 40 per cent of its graduated volume.

The volume in each container tested is not less than the nominal volume stated on the label as approved for the specific product.

(1) This chapter has undergone pharmacopoeial harmonisation. See chapter 5.8. Pharmacopoeial harmonisation.